MSCC

Multus Software Control Console

WPF Client User Manual

For Multus SDR Proficio / Geminus operators
Software version: MSCC WPF 8.4.x (and later 8.x)

# Contents

- 1. Introduction

- 2. System requirements

- 3. Installation and first-time setup

- 4. Starting and stopping a session

- 5. Main window overview

- 6. Button appearance (selected vs not selected)

- 7. Left panel — connection, audio, filters

- 8. Top panel — meters, VFOs, band bar

- 9. Right panel — operate controls

- 10. Tabs reference

- 11. Frequency calibration (FREQ CAL)

- 12. Noise tools: NB, NR, AN

- 13. Digital / DIG-U notes

- 14. Configuration files and reset

- 15. Troubleshooting

- 16. Glossary

# 1. Introduction

MSCC (Multus Software Control Console) is the PC application used to operate Multus SDR radios such as the Proficio and Geminus. This WPF edition is the modern Windows client: it talks to the radio stack over UDP and shows spectrum, meters, and day-to-day controls.

## What the client talks to

A normal session involves three backend programs plus the radio firmware:

- ms-sdr (or ms-sdr-MKII) — command hub, USB to the radio, keep-alive

- mscc-recv (sdrcore-recv) — receive DSP, spectrum/waterfall, NB/NR/AN

- Mscc-trans (sdrcore-trans) — transmit audio / digi path

- MSCC.Wpf.exe — this GUI

On a single PC you usually let MSCC launch the backends (Launch Servers checked). Advanced users can run backends separately and connect the GUI only.

[SCREENSHOT: Main window — full overview]

(Insert your capture here — crop to the relevant panel if possible.)

# 2. System requirements

| Item | Requirement |
| --- | --- |
| OS | Windows 10 or 11 (64-bit recommended) |
| Runtime | .NET 9 Desktop Runtime (Windows), if not bundled with your installer |
| USB | Radio connected and recognized (Proficio / Geminus USB) |
| Audio | Operator speaker + microphone configured (SETTINGS tab) |
| COM port | Configured when required by your station / CAT path |
| Install folder | Typical: C:\mscc-net9\ with client + server binaries together |
| Config data | %LocalAppData%\MSCC-NET9\ (created automatically on first run) |

# 3. Installation and first-time setup

## 3.1 Install layout

Your Multus package should place at least these items in one folder (example C:\mscc-net9\):

- MSCC.Wpf.exe and supporting DLLs (MSCC.Core, NAudio, and related)

- ms-sdr-MKII.exe (or the ms-sdr binary as shipped)

- mscc-recv.exe and Mscc-trans.exe (names as shipped)

- init-files\ (optional seed configs for first run / reset)

Keeping the GUI and subsystem binaries in the same folder lets the client discover backends the same way Multus packaging expects.

## 3.2 First launch checklist

- Connect the radio USB and power it on.

- Start MSCC.Wpf.exe.

- Open the SETTINGS tab.

- Set the COM port used by your station (if required).

- Set Operator speaker/microphone and Digital Out/Mic

- Confirm Server address is 127.0.0.1 for a local radio PC (or your remote host if backends run elsewhere).

- Leave Launch Servers checked for a normal single-PC station.

- Press Start (connection area).

## 3.3 Launch Servers vs connect-only

| Option | When to use |
| --- | --- |
| Launch Servers ON | Normal home station: MSCC starts ms-sdr, recv, and trans for you. |
| Launch Servers OFF | Backends already running (batch file, second PC, or headless appliance). GUI only connects. |
| Auto Start | Starts the session when the MSCC window loads (saved preference). |

When Launch Servers is ON and you press Stop (or exit after launching backends), MSCC sends a stop so those processes shut down cleanly. When Launch Servers was OFF, Stop/exit leave backends running.

# 4. Starting and stopping a session

## 4.1 Start

- Press Start when COM and audio requirements are satisfied.

- MSCC waits briefly for backends, claims a session with ms-sdr, then shows firmware (FW) and core versions.

- Only one GUI client may own a session at a time.

## 4.2 Stop

- Press Stop to end the radio session.

- If this app launched the backends, they are stopped as well.

## 4.3 Keep-alive

While connected, MSCC and ms-sdr exchange keep-alive messages. If the GUI does not hear the server for about 10 seconds after a short startup grace period, a warning appears. Choose Continue only if you know the stack is recovering; otherwise Stop and Start again.

# 5. Main window overview

Think of the window in four regions:

| Region | What you find there |
| --- | --- |
| Left | Start/Stop, server IP/port, Auto Start, Launch Servers, Phones/Digital audio, Lo/Hi/CW filters, zoom, RIT tools, temperatures |
| Top / center-top | S-meter, ALC, VFO A/B, frequency, radio model & band buttons, GEN |
| Center tabs | MAIN spectrum, CW, RX/TX, Favorites, QRP CAL, AMP CAL, RX IQ, TX IQ, FREQ CAL, SETTINGS |
| Right | Always-visible operate cluster: PTT, TUN, AMP, AGC, COMP, power, modes, NB/NR/AN, MON, TX BW, CW speed/pitch, LOG, versions |

# 6. Button appearance (selected vs not selected)

Most gold toggle buttons share one visual language across MSCC.  If custom colors are chosen, generally OFF is a brighter color and ON is darker by a few shades for on:

| State | Appearance | Meaning |
| --- | --- | --- |
| Not selected (off) | Bright gold face, black text | Feature or mode is off / not chosen |
| Selected (on) | Darker gold face, white text | Feature or mode is on / active |
| Alert latched (e.g. PTT / TUN / AMP) | Red-style alert face | Transmit or amp path engaged — treat with care |
| Disabled | Grayed / reduced opacity | Not available for current model or mode |

Examples: mode buttons (USB, LSB, CW…), NB / NR / AN, AMP, COMP, AGC cycle, and similar toggles.

# 7. Left panel — connection, audio, filters

## 7.1 Connection

- Server address / port — usually 127.0.0.1 for a local station PC.

- Start / Stop — session control.

- Auto Start — start when the window opens.

- Launch Servers — spawn backends with Start.

## 7.2 Audio path P / D

- P (Phones) — operator speaker and mic.

- D (Digital) — digital / VAC path for apps such as WSJT-X.

DIG-U mode pairs naturally with Digital audio.

## 7.3 Filters and zoom

- Low cut / High cut — cycle filter edges for voice-style modes.

- CW bandwidth — cycle CW filter widths.

- Step — VFO tune step.

- Zoom — display zoom of the panadapter (1×–4×) around the VFO; magnifies the view of the same RF data.

# 8. Top panel — meters, VFOs, band bar

## 8.1 Meters

- S-meter — receive signal strength.  With Multus SWR meter connected this changes to FWD Power during TX.

- ALC — transmit ALC indication.

## 8.2 VFO A / VFO B

- Click a VFO panel to make it active (mouse wheel tunes the active VFO).

- RIT offset may be shown per VFO when used.

## 8.3 Radio model button

Cycles Proficio ↔ Geminus (UI band enablement and waterfall banks). Match the model to your hardware:

- Proficio — HF bands; GEN rotates standard HF time/frequency beacons (WWV / CHU / RWM / USER style).

- Geminus — enables 2200 m / 630 m; GEN uses LF carriers useful for frequency calibration.

[SCREENSHOT: Band bar with Proficio/Geminus model button and GEN]

(Insert your capture here — crop to the relevant panel if possible.)

## 8.4 Band buttons

Click a band to QSY to a default or last-used frequency for that band. Gray (disabled) bands cannot be used for the selected radio model.

# 9. Right panel — operate controls

These stay visible while you use tabs, so you can work spectrum and still PTT or change mode.

## 9.1 Transmit cluster

| Control | Function |
| --- | --- |
| PTT | Push-to-talk (alert style when on). |
| TUN | Tune / carrier helper for calibration and matching. |
| AMP | External amplifier path / PA control (alert when engaged). |
| Drive / power | Context-sensitive drive: TUN, CW, SSB, or AM carrier depending on mode. |

## 9.2 AGC, compression, spectrum popup

| Control | Function |
| --- | --- |
| AGC (SLO / MED / FAST) | Cycles AGC speed. |
| AGC fast release | Fine timing for AGC release (ms). |
| COMP | Compression on/off (phones path). |
| Compression level | dB amount when COMP is active (phones / P). |
| S/W | Opens Spectrum / Waterfall settings popup. |

## 9.3 Modes

- LSB, USB, DIG-U, CW, AM — FM may appear disabled until implemented.

- DIG-U uses its own low/high cut memory and expects Digital (D) audio for many digi apps.

## 9.4 Noise tools and CW face controls

| Control | Function |
| --- | --- |
| NB | Noise Blanker on/off. Pulse and threshold sliders below when used. |
| NR | Noise Reduction on/off. Level slider when on. |
| AN | Auto Notch on/off (adaptive tone notch). Selected = on = darker gold. |
| MON | Monitor. |
| TX BW | Transmit bandwidth cycle. |
| CW speed / pitch | Quick CW face controls (full set also on CW tab). |
| LOG | Debug / monitor log window. |
| Versions | MSCC (this client), Core (ms-sdr), FW (radio firmware). |

# 10. Tabs reference

## 10.1 MAIN

Primary spectrum and waterfall display. Click in the panadapter to tune (subject to Auto Snap settings from the S/W popup). Most operating is done with MAIN visible plus the right panel.

## 10.2 CW

Electronic keyer and CW parameters: speed, pitch, keyer mode, spacing, paddle, weight, hold, QSK, phones, and related options. Use when setting up CW beyond the quick right-panel controls.

## 10.3 RX/TX

Receive/transmit preferences and power levels (tune / CW / SSB / AM). NB, NR, and AN also appear on the right operate panel for quick access while watching the spectrum.

## 10.4 FAVORITES

Client-only named memories per band. Example: save “FT8” on 20 m and a separate “FT8” on 15 m.

- Choose the band context (or use the radio’s current band).

- Enter a name → Save current conditions.

- Select a row → Recall or Delete.

Favorites are stored under %LocalAppData%\MSCC-NET9\ (MSCC_Favorites.ini).

## 10.5 QRP CAL

Transceiver / QRP power calibration. Requires AMP off. Select band and use the calibrate workflow and drive controls as labeled. Band lamps show client-side calibrated status for convenience. Follow Multus hardware notes for dummy load and power meter use.

## 10.6 AMP CAL

External amplifier calibration. Requires AMP on. Band-oriented workflow with amp-specific steps. Grayed when AMP is off.

## 10.7 RX IQ

Receive I/Q balance tools: start session, optional +24 kHz LO aid, offset, commit, apply-all, LO fine offset. Use carefully; incorrect I/Q settings can hurt image rejection.

## 10.8 TX IQ

Transmit I/Q balance (QRP / AMP off). Manual offset with tune carrier; commit when ready. Factory reset of I/Q tables is available with confirmation.

## 10.9 FREQ CAL

See the dedicated section below.

## 10.10 SETTINGS

- UI appearance (colors / chrome).

- COM port.

- Audio devices (operator and digital).

- Optional external Wi-Fi SWR meter.

- Configuration reset — reseeds configs from install init-files when present (does not wipe logs by design).

# 11. Frequency calibration (FREQ CAL)

Use FREQ CAL with a known standard (WWV, CHU, or LF carriers on the Geminus GEN list). Typical controls:

| Button | Purpose |
| --- | --- |
| LOOSE | Widens the peak-search window for a coarse check. |
| AUTO | Runs automatic calibrate sequence (coarse then fine stages). |
| MANUAL | Enters manual PPM adjustment mode. |
| CHECK | Measures error near the current frequency without a full auto run. |
| RESET | Resets calibration state as implemented. |
| PPM − / + | Manual PPM steps (rate-limited to protect the radio). |

Tips:

- Use a strong, known carrier; weak peaks can fail CHECK.

- Prefer GEN standards when available.

- Allow AUTO to finish; watch the status label and progress bar.

[SCREENSHOT: FREQ CAL tab]

(Insert your capture here — crop to the relevant panel if possible.)

# 12. Noise tools: NB, NR, AN

| Tool | Name | Selected (darker) means | Notes |
| --- | --- | --- | --- |
| NB | Noise Blanker | Blanker ON | Pulse width and threshold sliders below the buttons. |
| NR | Noise Reduction | Reduction ON | Level slider when NR is on. |
| AN | Auto Notch | Notch ON | Adaptive notch for steady tones/heterodynes; no extra level control. |

Visual rule (same as other toggles): bright gold = off; darker gold with white text = on. When AN is selected (on), the client sends auto-notch enable = 1 to the receive DSP.

# 13. Digital / DIG-U notes

- Select DIG-U mode.

- Set audio path to D (Digital).

- Point your digi app (for example WSJT-X) at the Multus digital devices configured in SETTINGS.

- Confirm TX drive on the right-panel power control for digi.

Exact device names depend on your PortAudio / virtual audio setup and Multus install package.

# 14. Configuration files and reset

Client preferences live primarily in:

%LocalAppData%\MSCC-NET9\

| File (examples) | Role |
| --- | --- |
| MSCC_Client.ini | UI, connection, power, spectrum/waterfall prefs |
| MSCC_LastUsed.ini / VFOB | Per-band last frequencies |
| MSCC_Favorites.ini | Favorites |
| user_controls.ini | Shared control snapshot used with ms-sdr |
| startup.ini | Last frequency/mode for start / appliance style resume |
| cw.ini | CW keyer parameters for ms-sdr |
| freq_cal.ini / power_cal.ini / … | Calibration-related data |
| logs\ | mscc.log, ms-sdr.log, sdrcore-recv/trans logs |

SETTINGS → configuration reset can delete client configs and reseed from install init-files, then restart MSCC. Back up the folder first if you care about custom calibrations or favorites.

# 15. Troubleshooting

| Symptom | What to try |
| --- | --- |
| Start disabled / setup incomplete | Set COM port and operator speaker/mic in SETTINGS. |
| No spectrum / no sound | Confirm Launch Servers, radio USB, and backends in Task Manager; check logs\. |
| Keep-alive lost | Stop, wait a few seconds, Start again; ensure only one client; check USB cable/power. |
| Second PC cannot connect | Only one session owner; stop the first client or run intentional connect-only design. |
| Wrong bands enabled | Set radio model (Proficio vs Geminus) on the band bar. |
| FREQ CAL CHECK fails | Stronger standard signal; try LOOSE; center near the carrier; try AUTO. |
| AN seems wrong | Selected (darker) = ON. Confirm sdrcore-recv log shows Enabled: 1 when selected. |
| After COM/audio change | Stop, then Start so backends pick up new devices. |

Log locations: %LocalAppData%\MSCC-NET9\logs\ — mscc.log (GUI), ms-sdr.log, sdrcore-recv.log, sdrcore-trans.log.

# 16. Glossary

| Term | Meaning |
| --- | --- |
| MSCC | Multus Software Control Console (this GUI). |
| ms-sdr | Main server process bridging GUI, DSP cores, and radio USB. |
| recv / trans | Receive and transmit DSP cores. |
| FW | Radio firmware version reported at connect. |
| Core | ms-sdr version string. |
| QRP CAL | Transceiver power calibration tab. |
| GEN | General coverage / standard frequency list button. |
| VAC | Virtual audio cable (or equivalent) for digi apps. |
| AN | Auto Notch. |

# Support and community

For Multus SDR group releases: include the version shown on the right panel as MSCC: x.y.z when asking questions. Attach relevant log snippets (not necessarily full multi-megabyte files) when reporting connect or keep-alive issues.

Document prepared for group distribution. Hardware calibration procedures remain subject to Multus hardware documentation and safe station practice (dummy loads, power limits, amplifier interlocks).